Name :- Theshiya Aksha

Section A: Scenario-Based Concept Application

- Explain how Deep Packet Inspection (DPI) at the Network and Transport layers of the OSI model allows a startup to isolate specific traffic bottlenecks causing slow internet speeds despite high available bandwidth.

A startup is facing slow internet speeds, even though bandwidth utilization is low. In this situation, Deep Packet Inspection (DPI) can be used to analyze traffic at the Network Layer (Layer 3) and Transport Layer (Layer 4) to identify the bottleneck.

At Network Layer (L3)

DPI examines:

Source IP address

Destination IP address

Protocol (ICMP, TCP, UDP)

Routing information

This helps identify:

Which devices or servers are generating excessive traffic.

Whether traffic is taking an inefficient route.

If specific IPs are causing congestion.

Example:
A backup server is continuously sending large amounts of data to a cloud server, consuming network resources.

At Transport Layer (L4)

DPI examines:

TCP/UDP ports

TCP sessions

Packet retransmissions

Latency and packet loss

This helps identify:

Applications using excessive bandwidth.

High packet retransmission rates.

Slow TCP connections due to network issues.

Example:
A video conferencing application on TCP port 443 is experiencing high packet loss, causing slow performance despite available bandwidth.

How DPI Isolates the Bottleneck

Captures and analyzes packets.

Groups traffic by IP, port, and protocol.

Measures latency, packet loss, and retransmissions.

Identifies which application, user, or device is causing delays.

Allows administrators to apply QoS, traffic shaping, or fix the problematic application.

2. Explain the distinct roles of DHCP and DNS in a scenario where startup devices are successfully assigned IP address.

-> DHCP (Dynamic Host Configuration Protocol) Role

DHCP automatically provides:

IP Address

Subnet Mask

Default Gateway

DNS Server Address

Example:
A laptop connects to the network and receives:

IP: 192.168.1.100

Subnet Mask: 255.255.255.0

Gateway: 192.168.1.1

DNS: 8.8.8.8

This allows the device to communicate on the network without manual configuration.

DNS (Domain Name System) Role

DNS translates human-readable names into IP addresses.

Example:

User enters www.google.com

DNS resolves it to an IP address such as 142.250.x.x

Browser then connects to that IP

Without DNS, users must access websites using IP addresses directly.

Scenario Analysis

Since devices are getting IP addresses:

✅ DHCP is functioning properly.

If websites cannot be opened by name:

DNS is failing.

Possible causes:

Incorrect DNS server configured in DHCP.

DNS server is down or unreachable.

Firewall blocking DNS traffic (Port 53 TCP/UDP).

Wrong DNS settings on client devices.

Troubleshooting Steps

Check IP configuration:

ipconfig /all      (Windows)

or

nmcli device show  (Linux)

Test internet connectivity:

ping 8.8.8.8

If successful, internet connectivity exists.

Test DNS resolution:

nslookup google.com

If this fails, DNS is the problem.

Verify DNS server settings and connectivity.

## 3.Explain the necessity of Inter-VLAN Routing and the specific role of static routes when a startup requires communication between two isolated departments on the same physical switch.

In a startup environment, different departments are often placed in separate VLANs to improve security and reduce unnecessary broadcast traffic. For example, the HR department may be assigned to VLAN 10 with the network 192.168.10.0/24, while the Finance department may be assigned to VLAN 20 with the network 192.168.20.0/24.

Although both departments are connected to the same physical switch, devices in VLAN 10 cannot directly communicate with devices in VLAN 20 because each VLAN represents a separate logical network and broadcast domain.

### Necessity of Inter-VLAN Routing

Inter-VLAN Routing is required to enable communication between devices located in different VLANs. A Layer 3 device, such as a router or Layer 3 switch, receives traffic from one VLAN and forwards it to the destination VLAN.

### Practical Scenario

Suppose:

- HR PC: 192.168.10.10 (VLAN 10)

- Finance PC: 192.168.20.10 (VLAN 20)

If the HR user tries to access a shared file on the Finance department's server, the communication will fail without Inter-VLAN Routing because the switch only forwards traffic within the same VLAN.

After configuring Inter-VLAN Routing (Router-on-a-Stick or Layer 3 Switch), the router becomes the default gateway for both VLANs and routes traffic between them.

### Role of Static Routes

Static routes are used when the router needs to know how to reach networks that are not directly connected.

For example, if:

- Router R1 handles VLAN 10 and VLAN 20.

- Another router R2 connects to the Startup's branch office network 192.168.30.0/24.

A static route can be configured on R1:

ip route 192.168.30.0 255.255.255.0 10.0.0.2

This tells R1 that traffic destined for 192.168.30.0/24 should be forwarded to the next-hop router 10.0.0.2.

In the case of communication between VLAN 10 and VLAN 20 on the same switch, Inter-VLAN Routing is the primary requirement. Static routes become necessary only when traffic must reach networks beyond the directly connected VLANs.

## 4. Explain the performance differences between ICMP and HTTP/HTTPS protocols that would cause a website to experience high latency while a ping test returns a fast, successful response.

When troubleshooting network performance in a startup environment, it is common to observe that a website loads slowly while a ping test to the same server shows a fast response. This occurs because ICMP and HTTP/HTTPS serve different purposes and operate differently.

### Role of ICMP

ICMP (Internet Control Message Protocol) is used for network connectivity testing and diagnostics. The ping command sends small ICMP Echo Request packets and measures the time taken to receive an Echo Reply.

For example:

ping google.com

Output:

Reply from 142.250.xx.xx: time=15ms

This indicates that the network path between the client and the server is reachable and responding quickly.

### Role of HTTP/HTTPS

HTTP and HTTPS are application-layer protocols used for loading web pages and transferring web content.

When a user opens a website, several additional processes occur:

- DNS name resolution

- TCP three-way handshake

- SSL/TLS handshake (for HTTPS)

- Web server processing

- Downloading HTML, CSS, JavaScript, images, and other content

Each of these steps can introduce delays even when the basic network connection is healthy.

### Practical Scenario

Suppose employees in a startup report that www.companyportal.com takes 10–15 seconds to load.

Network testing shows:

ping companyportal.com

Result:

Reply from server: time=12ms

The ping is fast, indicating that the server is reachable. However, the website remains slow.

Possible reasons include:

- Slow DNS resolution

- Overloaded web server

- High CPU or memory usage on the server

- Large images or files on the webpage

- Slow database queries

- SSL/TLS negotiation delays

- Network proxy or firewall inspection delays

### Why Ping Can Be Fast While Website Is Slow

| ICMP (Ping) | HTTP/HTTPS |
| --- | --- |
| Tests only network reachability | Loads complete web application |
| Uses small packets | Transfers large amounts of data |
| No TCP handshake | Requires TCP connection establishment |
| No SSL/TLS processing | HTTPS requires encryption setup |
| No server-side application processing | Depends on web server and database performance |
| - Explain the conceptual logic of Firewall Access Control Lists (ACLs) and the strategic placement of a firewall to restrict unauthorized traffic between a startup’s management and general staff departments. Firewall ACL (Access Control List) – Conceptual Logic A Firewall ACL is a set of rules that determines which network traffic is allowed or denied based on criteria such as: Source IP address Destination IP address Protocol (TCP, UDP, ICMP) Port number (80, 443, 22, etc.) Direction of traffic The firewall checks packets against ACL rules from top to bottom. As soon as a packet matches a rule, the corresponding action (Allow/Deny) is applied. Example ACL Logic Rule No. Source Destination Service Action 1 Staff Network Management Network Any Deny 2 Management Network Staff Network Any Allow 3 Staff Network Internet HTTP/HTTPS Allow 4 Any Any Any Deny How it works: Staff users cannot directly access management systems. Management users can access staff systems if required. Staff can browse the internet. Any traffic not explicitly allowed is blocked. Strategic Firewall Placement Startup Network Design Internet ¦ [Firewall] ¦ Core Switch / \ / \ Management VLAN Staff VLAN 192.168.10.0/24 192.168.20.0/24 Why Place the Firewall Here? The firewall sits between network segments and controls traffic moving between: Internet ↔ Internal Network Management Department ↔ Staff Department Benefits: Department Isolation Staff cannot access HR, Finance, or Management systems. Data Protection Sensitive company information remains protected. Access Monitoring All traffic is logged and audited. Attack Containment If a staff PC is compromised, the attacker cannot easily reach management systems. Example Scenario Networks Management VLAN: 192.168.10.0/24 Staff VLAN: 192.168.20.0/24 Security Requirement Staff should not access management PCs. Management can access staff PCs. Both can access the internet. Firewall Rules Deny 192.168.20.0/24 -> 192.168.10.0/24 Allow 192.168.10.0/24 -> 192.168.20.0/24 Allow Both VLANs -> Internet Deny Any Any ## Task 1: Design a VLSM subnet scheme for 3 departments requiring 15 hosts each using 192.168.10.0/24 ### Step 1: Calculate Required Hosts Each department requires 15 hosts. Formula: 2n−2≥152^n - 2 \geq 152n−2≥15 Checking: - 24−2=142^4 - 2 = 1424−2=14 ❌ - 25−2=302^5 - 2 = 3025−2=30 ✅ So, we need 5 host bits. Subnet mask: 32−5=2732 - 5 = 2732−5=27 Therefore, use /27 subnet mask. Subnet Mask: 255.255.255.224 Each /27 subnet provides: - Total IPs = 32 - Usable Hosts = 30 ## VLSM Allocation ### Department 1 Item; Value. Network Address; 192.168.10.0/27. First Host; 192.168.10.1. Last Host; 192.168.10.30. Broadcast; 192.168.10.31. ### Department 2 Item; Value. Network Address; 192.168.10.32/27. First Host; 192.168.10.33. Last Host; 192.168.10.62. Broadcast; 192.168.10.63. ### Department 3 Item; Value. Network Address; 192.168.10.64/27. First Host; 192.168.10.65. Last Host; 192.168.10.94. Broadcast; 192.168.10.95. ## Network Diagram (Packet Tracer) Router ¦ --------------- ¦ ¦ ¦ VLAN10 VLAN20 VLAN30 Dept A Dept B Dept C 192.168.10.0/27 192.168.10.32/27 192.168.10.64/27 ## Why /27? - Each department needs 15 hosts. - /28 provides only 14 usable hosts → insufficient. - /27 provides 30 usable hosts → sufficient and allows future growth. Task 2: Configure VLANs 10, 20, and 30 on a Cisco switch and assign physical interfaces to each. |  |

Step 1: Create VLANs

enable

configure terminal

vlan 10

name MANAGEMENT

vlan 20

name STAFF

vlan 30

name GUEST

exit

Step 2: Assign Interfaces to VLANs

Example:

VLAN 10 → Fa0/1 - Fa0/4

VLAN 20 → Fa0/5 - Fa0/8

VLAN 30 → Fa0/9 - Fa0/12

Assign VLAN 10

interface range fa0/1 - 4

switchport mode access

switchport access vlan 10

exit

Assign VLAN 20

interface range fa0/5 - 8

switchport mode access

switchport access vlan 20

exit

Assign VLAN 30

interface range fa0/9 - 12

switchport mode access

switchport access vlan 30

exit

Step 3: Save Configuration

end

write memory

or

copy running-config startup-config

Verification Commands

Check VLANs:

show vlan brief

Check Interface Status:

show interfaces status

Expected output:

VLAN Name                             Status Ports

10   MANAGEMENT                       active Fa0/1,Fa0/2,Fa0/3,Fa0/4

20   STAFF                            active Fa0/5,Fa0/6,Fa0/7,Fa0/8

30   GUEST                            active Fa0/9,Fa0/10,Fa0/11,Fa0/12

Packet Tracer Topology Example

PC1 ---- Fa0/1  (VLAN 10)

PC2 ---- Fa0/2  (VLAN 10)

PC3 ---- Fa0/5  (VLAN 20)

PC4 ---- Fa0/6  (VLAN 20)

PC5 ---- Fa0/9  (VLAN 30)

PC6 ---- Fa0/10 (VLAN 30)

Cisco Switch

Task 3: Set up a DHCP pool on a Router/L3 switch to provide dynamic addressing for the VLAN 10 subnet.

Step 1: Exclude Gateway IP

enable

configure terminal

ip dhcp excluded-address 192.168.10.1

Ye command DHCP ko gateway IP assign karne se rokta hai.

Step 2: Create DHCP Pool

ip dhcp pool VLAN10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

Step 3: Configure VLAN 10 Gateway

Agar L3 Switch hai:

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Agar Router-on-a-Stick hai:

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Step 4: Save Configuration

copy running-config startup-config

Verification Commands

Check DHCP Pool:

show ip dhcp pool

Check Assigned IPs:

show ip dhcp binding

Check VLAN Interface:

show ip interface brief

Packet Tracer Testing

PC ko VLAN 10 ke port par connect karo.

PC → Desktop → IP Configuration.

DHCP button par click karo.

Expected result:

IP Address: 192.168.10.x

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

DNS Server: 8.8.8.8

Task 4: Capture a network trace in Wireshark and identify the IP addresses in the DNS query and HTTP GET headers.

Step 1: Start Packet Capture

Open Wireshark.

Select the active network interface (Ethernet/Wi-Fi).

Click Start Capture (blue shark fin icon).

Step 2: Generate Traffic

DNS Traffic

Open Command Prompt:

nslookup google.com

or simply open a website in a browser.

HTTP Traffic

Visit a non-HTTPS site such as:

http://neverssl.com

(Most websites use HTTPS today, so HTTP GET packets may not be visible unless using an HTTP-only site.)

Step 3: Find DNS Query

In Wireshark filter:

dns

Select a DNS packet and look for:

Source: 192.168.10.100

Destination: 8.8.8.8

Standard query A google.com

Example:

Source IP (Client): 192.168.10.100

Destination IP (DNS Server): 8.8.8.8

Step 4: Find HTTP GET Request

Filter:

http.request.method == "GET"

Select the GET packet.

Example:

Source: 192.168.10.100

Destination: 34.117.59.81

GET / HTTP/1.1

Host: neverssl.com

Identify:

Source IP = Client IP

Destination IP = Web Server IP

HTTP Method = GET

Host Header = Website Name

Example Report

Protocol	Source IP	Destination IP	Details

DNS Query	192.168.10.100	8.8.8.8	Query for google.com

HTTP GET	192.168.10.100	34.117.59.81	GET / Host: neverssl.com

Wireshark Filters

dns

http

http.request.method == "GET"

ip.addr == 192.168.10.100

Section C: Integrated Case Study Scenario: TechLaunch, a startup with 40 employees, is experiencing "flapping" network connectivity. Employees in the Marketing wing report that their internet drops for 30 seconds every few minutes. The Engineering wing, located on a different VLAN, remains perfectly stable. A ping test from a Marketing PC to the Core Switch shows a consistent 25% packet loss and fluctuating latency. Questions:

- Based on the symptoms, identify the two most likely OSI layers where this fault is occurring.

Layer 1 – Physical Layer

Possible issues:

Faulty Ethernet cable

Loose cable connection

Damaged switch port

Bad NIC on Marketing PC

Symptoms:

Packet loss

Intermittent connectivity

Fluctuating latency

Layer 2 – Data Link Layer

Possible issues:

Switching loop

Broadcast storm

VLAN misconfiguration

STP issues

Symptoms:

Connectivity drops every few minutes

One VLAN affected while another VLAN remains stable

Packet loss to the Core Switch

Most likely: Layer 2 issue (Switching Loop/Broadcast Storm).

- Outline a 4-step "Bottom-Up" troubleshooting sequence to isolate this specific issue.

->  2. 4-Step Bottom-Up Troubleshooting

Step 1: Check Physical Connectivity

Verify:

Cable condition

Link LEDs

Patch panel connections

Switch port status

Commands:

show interfaces status

show interfaces fa0/1

Step 2: Check Interface Errors

Look for:

CRC errors

Input errors

Collisions

Drops

Command:

show interfaces fa0/1

If counters continuously increase, suspect cable or port issues.

Step 3: Check Layer 2 Health

Verify:

VLAN assignment

MAC table stability

STP status

Commands:

show vlan brief

show spanning-tree

show mac address-table

Step 4: Monitor Traffic Levels

Check for:

Broadcast storms

Excessive MAC learning

High CPU usage

Commands:

show interfaces counters

show processes cpu

If broadcast traffic is abnormally high, investigate switching loops.

- List three CLI commands you would run on the switch to check for cable faults or broadcast storms.

->

3. Three CLI Commands

Check Interface Errors

show interfaces

Look for:

CRC errors

Input errors

Output drops

Check Spanning Tree Status

show spanning-tree

Look for:

Frequent topology changes

Ports repeatedly transitioning states

Check Broadcast Traffic

show interfaces counters

or

show interfaces | include broadcast

Look for unusually high broadcast packets.

- If the issue is a "Switching Loop," explain which protocol should have prevented this and why it might have failed.

->  4. Switching Loop Scenario

Protocol That Should Prevent It

Spanning Tree Protocol (STP)

STP prevents Layer 2 loops by:

Detecting redundant paths

Blocking one of the redundant links

Creating a loop-free topology

Why It Might Have Failed

Possible causes:

STP disabled

no spanning-tree vlan 10

Misconfigured PortFast on switch-to-switch links

spanning-tree portfast

BPDU filtering enabled

spanning-tree bpdufilter enable

Rogue switch connected by a user

STP convergence problems

Result:

Broadcast storm

MAC table instability

Intermittent connectivity

Packet loss

- Propose a long-term configuration change to ensure an error in one department doesn't impact the rest of the office.

5. Long-Term Configuration Change

Network Segmentation Using VLANs

Separate departments:

VLAN 10 - Management

VLAN 20 - Engineering

VLAN 30 - Marketing

VLAN 40 - Guest

Benefits:

Broadcast traffic remains inside each VLAN.

Problems in Marketing do not affect Engineering.

Easier troubleshooting.

Better security.

Additional Protection

Enable:

spanning-tree mode rapid-pvst

storm-control broadcast level 5.00

spanning-tree bpduguard enable

errdisable recovery cause bpduguard